SECTION 098315

ACOUSTIC LOUVERED FENCE AND GATES

The specifier will need to edit this product specification for a specific project to reflect the options and applications being used.  This guide section has been written to show items to be determined reflected in italic red.  PalmSHIELD does not warrant that these specifications are complete and applicable to all applications.  It is the specifier’s responsibility to assure that use of PalmSHIELD is an appropriate fit for the site and conditions.

PART 1 – GENERAL

  - DESCRIPTION

- This section describes the following fence system:

- Acoustic Fixed louver modular fencing panels fabricated with extruded aluminum louvers, flat aluminum bars including extruded aluminum panel posts and aluminum louver gates. Louvered security fence and gates shall be furnished and installed as shown on the plans and specified herein, overall height of vertical louver framework shall be (state height here) tall.

  - REQUIREMENTS

- Furnish materials, labor, expertise and equipment necessary to complete all work specified in this section and as shown on the drawings.

- Structural Performance:  Provide product and installation capable of withstanding the effects of gravity lads and the following loads and stresses within limits and under conditions indicated.

    - Uniform pressure of 30 lbf/sq. ft. acting inward or outward.

    - Thermal Movements resulting from a temperature change (range) of 120 degrees Fahrenheit ambient  and 180 degrees Fahrenheit material surfaces.

- Sound Absorption Test Data per ASTM C423:

        - NRC: 0.85.

        - SAA: 0.87.

  - SUBMITTALS

- Shop drawings and manufacturer's literature:  Provide specifications and construction detail drawings to substantiate quality of materials and provide details of fabrication and installation.

- Submittals shall be in accordance with standard construction practices to include complete detailed layout of all panels, posts, gates.  Submittals shall include plan layout, elevations and section views of panels, posts and gates.

- Certificate:  manufacturer's certification that materials meet specification requirements.

  - REFERENCES

- ASTM B 209 – Standard Specification for Aluminum and Aluminum-Alloy Sheet 				and Plate.

- ASTM B 221 – Standard Specification for Aluminum and Aluminum-Ally 					Extruded Bars, Rods, Wire, Profiles and Tubes.

- ASTM D3363 – Standard Test Method for Film Hardness by Pencil Test.

- ASTM D2794 – Standard Test Method for Resistance of Organic Coatings to the 				Effects of Rapid Deformation.

- ASTM B117 – Standard Practice for Operating Salt Spray Apparatus.

- ASTM D822 – Standard Practice for Filtered Open-Flame Carbon-Arc Exposures of 			Paint and Related Coatings.

- AWS D1.2 Structural Welding Code – Aluminum.

- UL 94 - Surface Burning Characteristics: Acoustical product to perform as specified when tested in accordance with UL 94. Acoustical products surface burning performance should comply with the International Building Code and other local building code requirements.

- Acoustical Characteristics: Acoustical products to perform as specified when tested in accordance with ASTM E90.

  - QUALITY ASSURANCE

- Installation of fence and materials shall conform to the requirements of the fence manufacturer.

- The fence shall be warranted from any defects in materials and workmanship for a period as specified in the relevant section of the contract documents.

PART 2 – PRODUCTS

  - MATERIALS

- Louvered security fence:

  - Approved Louvered Fence System, Manufacturer:

    - Palm Shield Manufacturing or approved equal.

    - Silere Acoustic Louvered System

- Material Descriptions:

  - Extruded Aluminum: ASTM B 221, Alloy 6063 – Temper T-6.

  - Sheet Aluminum: ASTM B211, Alloy 6063 – Temper T6.

  - Powder Coating Material Hardness: ASTM 3363 2H.

  - Louver Panel Description:

    - Louver Panel Height: (state height here)

    - Louver Panel Width: Not to exceed 6’.

    - Louver fencing system consisting of horizontal, fixed louver, 	modular fence panels fabricated with extruded aluminum framing 	bars and supported by extruded aluminum fence posts.

    - Extended Flange to allow for a minimum of ½ inch overlap of louvers for 100% direct visual screening.

    - Fixed Louver Foils: 0.1250 inch thick louver with an installed profile of 2 inch wide X .625 inch tall filled with sound absorbing fire retardant foam.

    - Louver Vertical Framework: 3 inch x 3 inch x ¼ inch aluminum 	angle. Framework supporting the louvers shall be solid welded and 	mitered.

    - Louver Horizontal Top Cap: 3 inch x 3 inch x ¼ inch aluminum top 	cap.

    - Louver Horizontal Bottom Cap: 3 inch x 3 inch x ¼ inch angle 	bottom cap.

  - Micro Perforated Metal

    - Silk Metal by Acoustical Surfaces, Inc. (Basis of Design)

    - 123 Columbia Court North, Suite 201, Chaska, MN 55318.

    - 952-448-5300, Fax: 952-448-2613.

    - Toll Free: 1-800-448-3134. 2.

    - Description: Perforated metal shell and nonflammable acoustical material.

      - Shell: a. Material: [.040 aluminum] [galvanized steel] [stainless steel].

      - Perforation: 50% perforation pattern or as selected by Architect.

      - Finishes: Clear anodized aluminum

      - Size: As indicted on Drawings.

      - Attachment: Mechanical clips.

      - Applications: Walls, ceilings, steps, machine enclosures for indoor or outdoor.

      - Sound Absorption Test Data per ASTM C423:

        - NRC: 0.85.

        - SAA: 0.87.

  - Acoustical Panel Posts:

    - Panel posts shall be 4 inch square by ¼ inch minimum extruded tubular aluminum sections with solid aluminum caps. Length as specified on the contract drawings.

    - On center post spacing shall be as specified by manufacturer.

    - All fence posts to be plated with 10” x 10” x ¾” aluminum plates with four ¾” hole for anchors.

  - Fittings and accessories:  All fittings and accessories shall be stainless steel and sized as specified by the fence manufacturer. Fence panels to be attached to posts with ¼” x 1” stainless steel screws.  Panels and posts are predrilled to support level installation.

  - Anchor Bolts: Anchor bolts shall be (state means of anchoring posts to adjoining surface) and adequate to support loads based on screening height, exposures and loading.

  - Gates:  Swing to exterior of enclosure, size as shown on contract drawings.

    - Louver spacing, style and appearance shall be identical to fence panels.

    - Gate hinges to be Gorilla barrel hinge with ¾” rod, ball bearing, and grease zert.  Hinge plate to be ½” thick plates offset to create a 5/8” gap.  Standard hardware as required by the gate manufacturer for complete functional operation.  Hinges to be bolted to gate frame and field welded to steel gate posts.

    - Gate latch to be internal lock with exterior grab handles.  Lock may be keyed and rekeyed.  Lock is accessible from both sides of gate.

    - Welded frame, size as shown on the contract drawings, extruded 	aluminum tubing with aluminum fixed louver panels to match 	fencing material.

    - Drop rods to be 1” schedule 40 pipe and through bolted to gate frame.

    - Hardware: Size and type as determined by the manufacturer. Provide 	three hinges per leaf.

      - Provide 1 inch diameter center cane bolt assembly and strike, each door.

      - Provide padlockable slide bolt assembly.

    - Gate shall have welded frame fabricated from extruded aluminum 	tubing with aluminum fixed louver panels to match fencing material. 	Frame configurations shall be as indicated on the contract drawings.

    - Gate posts shall be as determined by manufacture.  Gate posts to be specified to support gates.

6.	Factory Finish: Aluminum fence panels, posts and gates shall receive 	polyester powder coating.

a.	Polyester powder coating: Electrostatically applied colored polyester 			powder coating heat cured to chemically bond finish to metal 			substrate.

b.	Color shall be as selected by Owner.

c.	Minimum hardness measured in accordance with ASTM D3363 2H.

d.	Direct impact resistance tested in accordance with ASTM D2794. 			Withstand 160 inch-pounds.

e.	Salt spray resistance tested in accordance with ASTM B117: No 			undercutting, rusting, or blistering after 500 hours in 5 percent salt 			spray at 95° F and 95% relative humidity after 1,000 hours, less than 			3/16 inches undercutting.

f.	Weatherability tested in accordance with ASTM D822: No film 			failure and 88 percent gloss retention after 1 year exposure in South 			Florida with test panels tilted 45°.

PART 3 – EXECUTION

  - INSPECTION

  - Verify that final grading in fence location is completed and without irregularities which will interfere with fence installation.  PalmShield is designed to be installed on a level surface.  Variations in height, slopes, stairs steeping shall be shown on contract drawings and on submittal drawings.

  - Field verify all fence dimensions and layout prior to commencing installation.

  - Do not commence work until unsatisfactory conditions have been corrected.

3.2	INSTALLATION

- Install fence in accordance with manufacturer’s installation instructions.

- Install fence plumb and level. Posts are plated and mounted to top of surface.

- Do not install bent, bowed or otherwise damaged panels. Remove damaged 	components from site and replace.

- Secure fence panels with stainless with ¼” x 1” stainless steel screws to fence posts.  	All posts and panels will be predrilled to support level installation.

- Gates

  - Install gates and adjust hardware for smooth operation