SECTION 32 31 13

PLAYGROUND SEMI PRIVATE COMPOSITE INFILL & ALUMINUM FRAMEWORK SCREENING AND GATES

The specifier will need to edit this product specification for a specific project to reflect the options and applications being used.  This guide section has been written to show items to be determined reflected in italic red.  PalmSHIELD does not warrant that these specifications are complete and applicable to all applications.  It is the specifier’s responsibility to assure that use of PalmSHIELD is an appropriate fit for the site and conditions.

PART 1 – GENERAL

  - DESCRIPTION

- This section describes the following fence system:

- Semi-private panels fabricated with Trex Transcend Series series approved equal planks and structural aluminum profiles including extruded aluminum fence posts and aluminum panel gates. Semi-private panel security screening and gates shall be furnished and installed as shown on the plans and specified herein, overall height of semi-private screening shall be (state height here) tall.

  - REQUIREMENTS

- Furnish materials, labor, expertise and equipment necessary to complete all work specified in this section and as shown on the drawings.

- Structural Performance:  Provide product and installation capable of withstanding the effects of gravity lads and the following loads and stresses within limits and under conditions indicated.

    - Uniform pressure of 30 lbf/sq. ft. acting inward or outward.

    - Thermal Movements resulting from a temperature change (range) of 120 degrees Fahrenheit ambient  and 180 degrees Fahrenheit material surfaces.

  - SUBMITTALS

- Shop drawings and manufacturer's literature:  Provide specifications and construction detail drawings to substantiate quality of materials and provide details of fabrication and installation.

- Submittals shall be in accordance with standard construction practices to include complete detailed layout of all panels, posts, gates.  Submittals shall include plan layout, elevations and section views of panels, posts and gates.

- Certificate:  manufacturer's certification that materials meet specification requirements.

  - REFERENCES

- ASTM B 209 – Standard Specification for Aluminum and Aluminum-Alloy Sheet 				and Plate.

- ASTM B 221 – Standard Specification for Aluminum and Aluminum-Ally 					Extruded Bars, Rods, Wire, Profiles and Tubes.

- ASTM D3363 – Standard Test Method for Film Hardness by Pencil Test.

- ASTM D2794 – Standard Test Method for Resistance of Organic Coatings to the 				Effects of Rapid Deformation.

- ASTM B117 – Standard Practice for Operating Salt Spray Apparatus.

- ASTM D822 – Standard Practice for Filtered Open-Flame Carbon-Arc Exposures of 			Paint and Related Coatings.

- AWS D1.2 Structural Welding Code – Aluminum.

- ASTM D-7032-04: Standard Specification for Establishing Performance Ratings        for Wood-Plastic Composite Deck Boards and Guardrail Systems (Guards or Handrails)

- ASTM International. B. ASTM D-7031-04: Standard Guide for Evaluating Mechanical and Physical Properties of Wood-Plastic Composite Products

- ASTM International C. ASTM E-84-01: Test Method for Surface Burning Characteristics of Building Materials

- ASTM D 570: Water Absorption of Plastics

- ASTM D 1761: Mechanical Fasteners in Wood

- ASTM D -1413-99: Test method for Wood Preservatives by Laboratory Soilblock Cultures

- ASTM C177: Standard Test Method for Steady-State Heat Flux Measurements and Thermal Transmission Properties by Means of the Guarded-Hot-Plate Apparatus

  - QUALITY ASSURANCE

- Installation of fence and materials shall conform to the requirements of the fence manufacturer.

- The fence shall be warranted from any defects in materials and workmanship for a period as specified in the relevant section of the contract documents.

PART 2 – PRODUCTS

2.0	PLAYGROUND DESIGNS ELEMENTS

All playground screening should include the following design elements to meet and exceed ASTM F2049  Playground Fencing.

A.	Smooth head stainless steel screws and fasteners.  No sharp bolts or screws.

B.	Standard 4” square steel posts with round beveled ¾” beveled plates.  No corners no sharp edges.

C.	Vehicle barrier protection available with 6” square steel posts, ¾” beveled plates and continuous W-beam guardrail.

D.	3” x 3” x ¼” structural aluminum angle framework screened panels.

E.	Solid infill 36” and lower to prevent climbing and visibility for smaller children three or younger.

F.	No openings greater than 1 3/4”.  No openings smaller than 1/2”.  These openings prevent climbing and openings too small.

G.	Gates with lock sets at 48” heights.

H.	Panic hardware, fire exit and alarmed gate opening devices are required.

I.	Gate closures required and factory installed.

  - MATERIALS

- Semi private panel security fence:

  - Approved Architectural Semi Private Screening System, Manufacturer:

    - PalmSHIELD Manufacturing

- Material Descriptions:

  - Extruded Aluminum: ASTM B 221, Alloy 6063 – Temper T-6.

  - Sheet Aluminum: ASTM B211, Alloy 6063 – Temper T6.

  - Powder Coating Material Hardness: ASTM 3363 2H.

  - Composite Decking consisting of recycled Linear Low Density Polyethylene (LLDPE) and recycled wood. The product is extruded into shapes and sizes as follows:  i. Trex Transcend and Trex Enhance Decking Boards; 1 x 5.5”. ii. Lengths – 12, 16, and 20 feet iii. Color – To be specified by owner from Trex’ standard list of colors or approved equal.

  - Panel Description:

    - Panel Height: (state height here)

    - Panel Width: Not to exceed 60”

    - Semi private panel system consisting of composite modular panels fabricated with extruded aluminum framing structural profiles and supported by extruded aluminum fence posts.

    - Panel Vertical Framework: 2 ½” x 2 ½”  inch x 1/8”  inch aluminum angle. Framework supporting the semi private composite screening shall be solid welded and mitered.

    - Panel Horizontal Top Cap: 2 ½” x 2 ½”  inch x 1/8”  inch aluminum angle

    - Panel Horizontal Bottom Cap: 2 ½” x 2 ½”  inch x 1/8”  inch aluminum angle

  - Fence Posts:

    - Panel posts shall be 3” square x 1/8” inch minimum extruded tubular aluminum sections with solid aluminum caps. Length as specified on the contract drawings.

    - On center post spacing shall be as specified by manufacturer.

    - All fence posts to be plated with 8” x 8” x5 /8” aluminum plates with four ¾” hole for anchors.

  - Fittings and accessories:  All fittings and accessories shall be stainless steel and sized as specified by the fence manufacturer. Fence panels to be attached to posts with ¼” x 1” stainless steel screws.  Panels and posts are predrilled to support level installation.

  - Anchor Bolts: Anchor bolts shall be (state means of anchoring posts to adjoining surface) and adequate to support loads based on screening height, exposures and loading.

  - Gates:  Swing to exterior of enclosure, size as shown on contract drawings.

    - Panel spacing, style and appearance shall be identical to fence panels.

    - Gate hinges to be Gorilla barrel hinge with ¾” rod, ball bearing, and grease zert.  Hinge plate to be ½” thick plates offset to create a 5/8” gap.  Standard hardware as required by the gate manufacturer for complete functional operation.  Hinges to be bolted to gate frame and field welded to steel gate posts.

    - Gate latch to be internal lock with exterior grab handles.  Lock may be keyed and rekeyed.  Lock is accessible from both sides of gate.

    - Welded frame, size as shown on the contract drawings, extruded aluminum tubing with composite infill to match..

    - Drop rods to be 1” schedule 40 pipe and through bolted to gate frame.

    - Hardware: Size and type as determined by the manufacturer. Provide 	three hinges per leaf.

      - Provide 1 inch diameter center cane bolt assembly and strike, each door.

      - Provide padlockable slide bolt assembly.

    - Gate shall have welded frame fabricated from extruded aluminum tubing with composite infill to match. Frame configurations shall be as indicated on the contract drawings.

    - Gate posts shall be as determined by manufacture.  Gate posts to be specified to support gates.

6.	Factory Finish: Vinyl fence planks and aluminum posts and gates shall receive polyester powder coating.

a.	Polyester powder coating: Electrostatically applied colored polyester 			powder coating heat cured to chemically bond finish to metal 			substrate.

b.	Color shall be as selected by Owner.

c.	Minimum hardness measured in accordance with ASTM D3363 2H.

d.	Direct impact resistance tested in accordance with ASTM D2794. 			Withstand 160 inch-pounds.

e.	Salt spray resistance tested in accordance with ASTM B117: No 			undercutting, rusting, or blistering after 500 hours in 5 percent salt 			spray at 95° F and 95% relative humidity after 1,000 hours, less than 			3/16 inches undercutting.

f.	Weatherability tested in accordance with ASTM D822: No film 			failure and 88 percent gloss retention after 1 year exposure in South 			Florida with test panels tilted 45°.

PART 3 – EXECUTION

  - INSPECTION

  - Verify that final grading in fence location is completed and without irregularities which will interfere with fence installation.  PalmShield is designed to be installed on a level surface.  Variations in height, slopes, stairs steeping shall be shown on contract drawings and on submittal drawings.

  - Field verify all fence dimensions and layout prior to commencing installation.

  - Do not commence work until unsatisfactory conditions have been corrected.

3.2	INSTALLATION

- Install fence in accordance with manufacturer’s installation instructions.

- Install fence plumb and level. Posts are plated and mounted to top of surface.

- Do not install bent, bowed or otherwise damaged panels. Remove damaged 	components from site and replace.

- Secure fence panels with stainless with ¼” x 1” stainless steel screws to fence posts.  	All posts and panels will be predrilled to support level installation.

- Gates

  - Install gates and adjust hardware for smooth operation